| SUPERVISORY RESPONSIBILITIES | YES/NO |
| --- | --- |
| - Supervisors shall identify and report all job areas and locations that are or may be confined spaces. |  |
| - Maintain and provide a list of confined spaces that are identified which shall be submitted to the Safety Organization. |  |
| - In addition, the designated supervisors hall carry out the following tasks: |  |
| - Classify confined spaces as "permit required," “Alternate Procedure” or "non-permit required.” |  |
| - Identify personnel who will enter confined spaces. |  |
| - Identify the personnel under their supervision required to wear respirators. |  |
| - Advise personnel on routine measurement of respiratory hazards in confined spaces. |  |
| - Provide detailed instruction and training on confined space hazards and entry procedures to those who may enter confined spaces. |  |
| - Provide instruction to personnel on the proper use of equipment required for confined space entry. |  |
| - Maintain equipment that is used to enter confined spaces. |  |
| - Conduct work site inspections to review unit compliance with confined space entry procedures. |  |
| - Maintain records of equipment maintenance and employee training. |  |
| - Inform employees who may enter the permit confined space by posting danger signs or by training. |  |
| - Issuance and cancellation of entry permits. |  |
| - Establishment of a lockout program for their department. |  |
| - Identify and evaluate the hazards of permit spaces before employees enter them. |  |
| - Conduct a pre-entry briefing to inform entrants of possible hazards that may be encountered. |  |
| - Identify the people who will enter the confined spaces. |  |
| - Take the necessary measures to prevent entrance into prohibited permit spaces. |  |

| DUTIES OF THE “LEAD WORKER” (ENTRY SUPERVISOR): | YES/NO |
| --- | --- |
| - Know the potential hazards during entry and work. |  |
| - Determine if acceptable entry conditions are present at a permit space where entry is planned. |  |
| - Terminate entry as required by the standard. |  |
| - Verify that rescue services are readily available and the means for summoning them are operable. |  |
| - Remove unauthorized individuals who enter or try to enter the permit space during entry and work. |  |
| - Determine that entry and work operations remain consistent with entry permit terms and that acceptable entry conditions are maintained. |  |
| - Determine if entry into confined space is necessary to perform work. |  |
| Note: The person authorizing the entry may also serve as the entrant or attendant for the entry |  |

| The following minimum required equipment should be on hand: | YES/NO |
| --- | --- |
| - Ventilation, |  |
| - Barrier and warning signs, |  |
| - Gas monitor capable of measuring concentrations of oxygen, flammable gases, hydrogen sulfide and carbon monoxide. |  |
| - Eliminate any unsafe conditions before the access door or cover is opened. |  |

| Conduct hazard assessment | YES/NO |
| --- | --- |
| - Test the real or potential atmospheric hazards |  |
| - Oxygen content less than 19.5% or greater than 23.5% |  |
| - Flammable gases and vapors greater than 10% of the LEL (Lower Explosive Limit) |  |
| - Hydrogen Sulfide concentrations greater than 10 ppm (Parts per million) |  |
| - Carbon Monoxide concentrations greater than 35 ppm |  |

| Review the space for other observable serious safety and health hazards: | YES/NO |
| --- | --- |
| - mechanical, |  |
| - electrical, |  |
| - burn, |  |
| - heat stress, |  |
| - engulfment, or |  |
| - entrapment hazards, etc. |  |

| If any hazardous atmosphere exists, do the following: | YES/NO |
| --- | --- |
| - If possible, determine and eliminate the source of the atmospheric hazards (for example: carbon monoxide from nearby truck or gas-powered generator). |  |
| - When the atmosphere contains toxins or flammables, ventilate the space by drawing air out until the air has been changed over several times. |  |
| - When oxygen deficient, ventilate by pushing air into the space until the air has been changed over several times. |  |
| - Verify the hazardous atmosphere has been eliminated by testing the air. |  |

| Determine from information gathered above which of the following entry procedures is appropriate: | YES/NO |
| --- | --- |
| - Non-Permit Space - If there are neither real nor potential atmospheric hazards and no observable serious safety and health hazards, this should be certified in writing. |  |
| - Alternative Entry Procedures – If no observable serious safety and health hazards exist and atmospheric hazards are controlled with continuous ventilation; this should be certified in writing. |  |
| - Permit-Required Space - If there are any observable serious safety/health hazards in addition to potential or real atmospheric hazards, all procedure here must be followed. Authorize permit with signature. |  |
| - Non-Respirable Atmospheres - If hazardous atmosphere cannot be eliminated by continuous ventilation, contact the Safety Organization before continuing. |  |

| Follow pre-entry precautions: | YES/NO |
| --- | --- |
| - Notify affected departments of service interruption. |  |
| - Lock-out/tag-out all sources of energy (e.g. steam, electric, mechanical) posing a risk to workers. |  |
| - Install blank in affected pipes where valves are not secure or seated. |  |
| - Clean and/or purge any chemical storage vessel. |  |
| - Wear appropriate personal protective and respiratory protection. |  |
| - Have lights and or ladder available. |  |
| - If coordination is needed with contractors, see Contractor Checklist. |  |
| - Have appropriate MSDS's (Material Safety Data Sheet). |  |
| - Determine how often air monitoring will be conducted. |  |

| Additional precautions necessary for Permit-Required Spaces: | YES/NO |
| --- | --- |
| - Determine start and end times for authorized entry. |  |
| - Assign roles and responsibilities as entrant(s), attendant(s), leadworker(s) |  |
| - Set up non-entry rescue equipment (tri-pod, harness). |  |
| - Identify rescue service. |  |
| - Determine communication method between entrant/attendant. |  |
| - Conduct pre-entry briefing: review hazards, procedures, and precautions. |  |
| - Sign and post the Permit/Certification at the site. |  |
| - Continually ventilate the space by pushing air so that a positive pressure changes the air over several times every hour. Direct the clean air toward the worker. |  |
| - Test the air periodically while personnel are in the confined space to ensure the ventilation is preventing any accumulation of a hazardous atmosphere. |  |
| - Under the following conditions, personnel must exit the confined space, re-evaluate hazards, and modify entry procedures: |  |
| - If any hazardous atmosphere is detected after entry. |  |
| - If any health or safety hazard develops which was not anticipated. |  |
| - If Attendant (on Permit-Required Confined Space Entry) cannot effectively perform duties. |  |
| - If personnel in confined space are experiencing symptoms from heat stress or over-exposure to atmospheric hazards. |  |
| - When work is completed, return the space to original condition. Close out the permit/certification and submit the completed paperwork to your supervisor. |  |